## Supplementary Material

## Modelling mercury dynamics in the food web of the Augusta Bay

- Description of the Phytoplankton model

The dynamics of mercury concentration in phytoplankton biomass has been reproduced using the kinetic model devised for phytoplankton, i.e. the Phytoplankton model (Hendriks and Heikens, 2001; Radomisky and Ciffroy, 2015a). Specifically, we analyze the behaviour of the two main mercury species present in phytoplankton biomass, i.e. and , which describe the IHg and MeHg concentration at the position (x,y,z) of 3D domain of Augusta Bay and at a specific M. barbatus age (t’).

The mercury contents in phytoplankton depend strongly on the concentrations of the respective mercury species (HgII and MeHg) dissolved in seawater, which are in turn calculated by the HR3DHG model (Denaro et al., 2020). Accordingly, the mercury contents in phytoplankton are calculated in each node (x,y,z) of the same lattice used for the HR3DHG model. In the case study of Augusta Bay we used a 3D grid, constituted by a mesh of 10 and 18 elements regularly spaced of  454.6 m in both x- and y-direction, and with a variable number of vertical layers of 5 m depth in the z-direction.

The model is coded in C++ and adopts the forward Euler method for temporal discretization. Since the Phytoplankton model is coupled in real time with the HR3DHG model, the same time step (300 sec) has been fixed for both models. Specifically, the time step was chosen to guarantee the convergence and stability conditions associated with the numerical methods adopted for the HR3DHG model. The methods adopted for the spatial discretization of PDEs of HR3DHG model are reported by Denaro et al. (2020). The Phytoplankton model provides solutions in non-steady conditions because the spatial distributions of [HgII] and [MeHg] are obtained at non-steady state (Denaro et al., 2020).

The dynamics of  and  is modelled by considering three biological processes (Hendriks and Heikens, 2001; Radomisky and Ciffroy, 2015a): i) absorption of the two mercury species through the phytoplankton cell membrane (uptake term); ii) elimination of the two mercury species through the phytoplankton cell membrane (excretion term); iii) elimination of the two mercury species for dilution through the size growth of phytoplankton cell (growth term).

The mercury absorption process is described as an uptake term with a first-order kinetic. In particular, the uptake rate constant for both mercury species is considered inversely proportional to the sum of water layer resistance, lipid layer permeation influx resistance and flow delay, and  directly proportional to the average phytoplankton cell weight with exponent -κ. The water layer resistance and the flow delay are assumed constant, while the lipid layer resistance is defined as a function of mercury concentration in seawater by following Hendriks and Heikens (2001).

The mercury elimination processes in the phytoplankton cells are described by excretion and growth terms, both of which are regulated by first-order kinetics. For both investigated mercury species, the excretion rate constant is defined directly proportional to the average phytoplankton cell weight with exponent -κ, and inversely proportional to the dry fraction of phytoplankton, the dry tissue-water partition ratio, and the sum of water layer resistance, lipid layer permeation efflux resistance and flow delay (Hendriks and Heikens, 2001). The growth rate constant is defined directly proportional to the temperature correction factor, the biomass production coefficient and the average phytoplankton cell weight with exponent -κ, all parameters whose values are set as constant in agreement with the empirical studies performed by Hendriks et al. (2001).

Thus, the kinetic model for the mercury concentrations in phytoplankton biomass is defined by the following ordinary differential equations (Hendriks and Heikens, 2001; Hendriks et al., 2001; Radomisky and Ciffroy, 2015a):

(S1)

(S2)

where  and  are the IHg and MeHg concentrations in phytoplankton biomass, respectively ;  and  are the rate constants for the uptake from the seawater of IHg and MeHg, respectively ; [HgII] and [MeHg] are the concentrations of IHg and MeHg in seawater, respectively ;  and  are the rate constants for the excretion with seawater of IHg and MeHg, respectively [d-1];  is the dilution rate constant associated with phytoplankton biomass growth [d-1].

The mercury concentrations in seawater ([HgII] and [MeHg]) are obtained by solving in real time the PDEs of the HR3DHG model (Denaro et al., 2020). All rate constants are calculated according to Hendriks and Heikens (2001), assuming an analogue behaviour of MeHg compared with inorganic compounds. Thus, the rate constants are defined as follows:

;   	      	         (S3)

;  	         (S4)

(S5)

Here,  is the average phytoplankton cell weight ; is the rate exponent [dimensionless];  is the water layer diffusion resistance ;  is the lipid layer permeation influx resistance ;  and  are the lipid resistance exponents for IHg and MeHg, respectively [dimensionless];  is the water absorption-excretion coefficient ;  and  are the dry tissue-water partition ratios for IHg and MeHg, respectively ;  is the dry fraction of phytoplankton ;  is the lipid layer permeation efflux resistance ;  is the temperature correction factor [dimensionless];  is the biomass reproduction coefficient  .

All these parameters are obtained according to the available literature (Hendriks and Heikens, 2001; Hendriks et al., 2001), and are reported in Table S3. The average phytoplankton cell weight () is calculated by the average phytoplankton cell volume using the method of Radomisky and Ciffroy, (2015a). Assuming that the most representative phytoplankton population in terms of biomass is that of picoeukaryotes (Brunet et al., 2006; Brunet et al., 2007), we used their average cell volume (Strickland, 1960) for obtaining average phytoplankton cell weight. The rate exponent (), the water layer diffusion resistance (), the water absorption-excretion coefficient (), the temperature correction factor () and the biomass reproduction coefficient () are fixed according to Hendriks et al. (2001), while the lipid layer permeation influx resistance () and the lipid layer permeation efflux resistance () are fixed according to Hendriks and Heikens (2001). The lipid resistance exponents ( and ) and the dry tissue-water partition ratios ( and ) are calibrated, in accordance with the empirical findings of Hendriks and Heikens (2001), to obtain the best fit between the model results and experimental data for the mercury concentrations in marine organisms. The dry fraction of phytoplankton () is calculated by the average phytoplankton cell weight using the empirical equation by Hendriks and Heikens (2001) for the inorganic substances.

Eqs. (S1)-(S5) represent the model used for calculating the IHg and MeHg concentrations in phytoplankton biomass, which are used as input in the Invertebrate modules for decapoda, mysidacea, bivalvia and amphipoda. Finally, it is to be noticed that equations (S1)-(S2) are not affected the initial condition for mercury concentrations in phytoplankton biomass.

- Description of the Invertebrate model

The dynamics of mercury concentration in the main six preys of M. barbatus, has been reproduced using the kinetic model devised for invertebrates, i.e. the Invertebrate model (Hendriks and Heikens, 2001; Radomisky and Ciffroy, 2015b). Specifically, we investigated the behaviour of the two main mercury species present in the body of invertebrate populations, i.e. and , which denote the IHg and MeHg concentrations in i-th invertebrate population (M. barbatus prey) at the position (x,y) of the seabed of Augusta Bay and at a specific M. barbatus age (t’).

In general, the mercury concentrations in all six M. barbatus preys (invertebrates) depend strongly on the mercury concentrations both in the ingested marine organisms (phytoplankton, zoobenthos, etc.) and the assimilated detritus, while they are weakly affected by mercury species (HgII and MeHg) dissolved in seawater, which are calculated by the HR3DHG model (Denaro et al., 2020). Since the M. barbatus feeds on preys living in the seabed of coastal areas, the mercury concentrations in invertebrates are calculated in each node (x,y) of the same 2D grid used in the  HR3DHG model for the sediment surface layer of Augusta Bay, i.e. a mesh of 10 and 18 elements regularly spaced of  454.6 m in both x- and y-direction.

The Invertebrate model is coded in C++ and adopts the forward Euler method for temporal discretization. Since the six Invertebrate modules are coupled in real time both with the biogeochemical modules (HR3DHG model) and the Phytoplankton module, the same time step (300 sec) has been fixed for all modules. The Invertebrate model gives solutions in non-steady conditions because the spatial distributions of mercury concentrations in seawater, suspended particulate matter (SPM), sediments and phytoplankton are obtained at non-steady state (Denaro et al., 2020).

The dynamics of  and  is modelled by considering five biological processes (Hendriks and Heikens, 2001; Radomisky and Ciffroy, 2015b): i) absorption of the mercury through the invertebrate respiratory system (uptake term); ii) absorption of the mercury incorporated in food and detritus through the invertebrate gastro-intestinal tract (GIT) (ingestion terms); iii) elimination of the mercury  through the invertebrate respiratory system (excretion term); iv) elimination of the mercury incorporated in food and detritus through the invertebrate gastro-intestinal tract (GIT) (egestion term); v) elimination of the mercury for dilution through the size growth of invertebrate (growth term).

In the Invertebrate model, the mercury absorption process for respiration is described by an uptake term with a first-order kinetic, as well as for the Phytoplankton model. Here, the uptake rate constant for both mercury species is considered inversely proportional to the sum of water layer resistance, lipid layer permeation influx resistance and flow delay, and directly proportional to the average invertebrate weight with exponent -κ. Moreover, also in the Invertebrate model the water layer resistance and the flow delay are set as constant, while the lipid layer resistance is defined as a function of mercury concentration in seawater in accordance with the empirical studies by Hendriks and Heikens (2001).

The mercury assimilation process in the GIT is reproduced for each invertebrate population through different ingestion terms with a first-order kinetic, i.e. one term for each diet preference. In particular, the ingestion term of i-th invertebrate for  j-th prey is directly  proportional to (i) the diet preference of  i-th invertebrate for j-th prey, (ii) the assimilation rate constant of i-th invertebrate and (iii) the mercury content of  j-th prey. The diet preferences are fixed as constant in accordance with the experimental findings reported in previous works (Catalano et al., 2014; Ausili et al., 2020). The assimilation rate constant of i-th invertebrate for each mercury species is defined as a function of the efficiency for dietary assimilation, the temperature correction factor, the food ingestion coefficient and the average invertebrate weight with exponent -κ, in accordance with previous works (Hendriks and Heikens, 2001; Hendriks et al., 2001). The mercury contents of each prey are calculated by using the HR3DHG model, the Phytoplankton model and/or the Invertebrate model (Denaro et al., 2020; Hendriks and Heikens, 2001; Radomisky and Ciffroy, 2015a; Radomisky and Ciffroy, 2015b).

The mercury elimination processes in the invertebrate are described by excretion, egestion and growth terms, which are regulated by first-order kinetics. Similarly to the Phytoplankton model, the excretion rate constant for MeHg (IHg) is defined directly proportional to the average invertebrate weight with exponent -κ, and inversely proportional to the dry fraction of invertebrate, the dry tissue-water partition ratio for the MeHg (IHg), and the sum of water layer resistance, lipid layer permeation efflux resistance and flow delay. All parameters used for calculating the excretion rate constants are fixed as constant in accordance with the empirical findings on the inorganic substances reported by Hendriks and Heikens (2001). The egestion rate constant for MeHg (IHg) is defined directly proportional to the average invertebrate weight with exponent -κ, and inversely proportional to the dry fraction of invertebrate, the dry tissue-water partition ratio for the MeHg (IHg), and the sum of (i) the water layer diffusion resistance experienced during exchange with invertebrate food, (ii) the lipid layer permeation efflux resistance corrected by temperature factor and (iii) the digestion flow delay. We recall that, also in this case,  the egestion rate constants are calculated by using the parameters estimated empirically in previous works (Hendriks and Heikens, 2001; Hendriks et al., 2001).

According to Hendriks and Heikens (2001), also for invertebrates the growth rate constant is described as a function directly proportional to the temperature correction factor, the biomass production coefficient and the average invertebrate weight with exponent -κ, all parameters whose values are fixed as constant in the modelling of Hendriks et al. (2001).

Thus, the kinetic model for the mercury concentrations in i-th invertebrate population is defined by the following ordinary differential equations (Hendriks and Heikens, 2001; Hendriks et al., 2001; Radomisky and Ciffroy, 2015b):

(S6)

(S7)

where  and  are the IHg and MeHg concentrations in i-th invertebrate population, respectively ;  and  are the respiratory uptake rate constants of i-th invertebrate population for IHg and MeHg, respectively ;  and are the concentrations of IHg and MeHg in the deepest layer of seawater (z=zb), respectively ;  and  are the assimilation rate constants of i-th invertebrate population for IHg and MeHg, respectively ; ,  and  are the diet preferences of i-th invertebrate population for phytoplankton, SPM and sediments [dimensionless];  are the diet preferences of i-th invertebrate population for j-th prey [dimensionless]; , ,  and  are the IHg concentrations in phytoplankton,  j-th prey, SPM and sediments, respectively ; , ,  and  are the MeHg concentrations in phytoplankton,  j-th prey, SPM and sediments, respectively ;  and  are the excretion rate constants of i-th invertebrate population for IHg and MeHg, respectively ;  and  are the egestion rate constants of i-th invertebrate population for IHg and MeHg, respectively ;  is the dilution rate constant associated with the size growth of i-th invertebrate population .

To reproduce the mercury concentrations in i-th invertebrate population ( and ), as first step we calculate the mercury concentrations in seawater ([HgII] and [MeHg]) , SPM ( and ) and sediments ( and ) by solving the PDEs of the HR3DHG model (Denaro et al., 2020). Afterwards, we calculate the mercury concentrations in phytoplankton ( and ), prey invertebrates ( and ) and predator invertebrates ( and ) by solving, in real time with the biogeochemical modules, the ODEs of the Phytoplankton model and the Invertebrate modules, respectively (Hendriks and Heikens, 2001; Radomisky and Ciffroy, 2015a; Radomisky and Ciffroy, 2015b).

As well as for the Phytoplankton model, all rate constants of the Invertebrate model are calculated according to Hendriks and Heikens (2001), assuming an analogue behaviour of MeHg compared with inorganic compounds. According to this, the rate constants of the Invertebrate model are given by following equations:

;   	      	         (S8)

;  				         (S9)

;  		       (S10)

;

(S11)

(S12)

Here,  is the average weight of i-th invertebrate ; is the rate exponent [dimensionless];  is the water layer diffusion resistance ;  is the lipid layer permeation influx resistance ;  and  are the lipid layer resistance exponents for IHg and MeHg, respectively [dimensionless];  is the water absorption-excretion coefficient ; and  are, respectively, the efficiencies for dietary assimilation of IHg and MeHg by i-th invertebrate population [dimensionless];  is the temperature correction factor [dimensionless];  is the food ingestion coefficient ;  and  are the dry tissue-water partition ratios for IHg and MeHg, respectively ;  is the dry fraction of i-th invertebrate population ;  is the lipid layer permeation efflux resistance ;  is the water layer diffusion resistance for assimilation of chemicals from food ; is the dry fraction of food for i-th invertebrate ;  is the fraction of ingested food assimilated by i-th invertebrate [dimensionless];  is the biomass reproduction coefficient  . The model parameters are reported in Table S4.

All parameters with the exception of  and are fixed in accordance with empirical results obtained by other authors (Hendriks and Heikens, 2001; Hendriks et al., 2001). The average weights of all six invertebrate populations are set according to previous works (Radomisky and Ciffroy, 2015b; Campfens and Mackay, 1997; McKinney et al., 2004; Yakovlev and Yakovleva, 2010). The dry fraction of the food of each i-th invertebrate is calculated as follows:

(S13)

where  .

The lipid resistance exponents ( and ) and the dry tissue-water partition ratios ( and ) are the same used in the Phytoplankton model, and are calibrated in order to better reproduce the mercury concentrations observed in marine organisms of the Augusta Bay.

Eqs. (S6)-(S13) describe the model exploited for calculating the IHg and MeHg concentrations in each invertebrate population. These represent the paramount inputs for the Fish model, which is used for obtaining the dynamics of mercury concentrations in M. barbatus of the Augusta Bay. Finally, it is worth to underline that equations (S6)-(S7) are not affected the initial condition for mercury concentrations in invertebrates.

- Methods for the conversion of MeHg and IHg burdens into MeHg and IHg concentrations

To compare the model results with the experimental findings, the theoretical burdens of MeHg and IHg for humans are converted into MeHg and IHg concentrations, by using the values of the main organs and biological matrices weights (or volumes), estimated empirically in previous studies (Brochot 2015; Lewis 1956; Wright 2018; Slezak 2017). For this aim, we calculate the weights of major organs (including the blood) for both genders by using the same empirical equations of the Human model of Merlin-Expo (Brochot 2015). Here, the density of blood and main organs was fixed equal to 1 Kg/l (Brochot 2015). Afterwards, we calculate the [MeHg] and [IHg] in major organs (including the blood) by dividing the MeHg and IHg burdens of each organ by its weight (or volume). Similarly, the [MeHg] and [IHg] for the whole body are obtained by dividing the MeHg and IHg burdens to the body weight (BDW(t)).

The instantaneous [IHg] in urine ([IHgU]) is calculated as follows:

IHgU(t)=Uit-Uit-1VU∙dt  (S14)

where Uit and Uit-1 are the IHg burdens in urine at time t and t-1, respectively; VU is the daily volume of urine, which is set at 1.728 l∙d-1 in accordance with the experimental findings acquired by Lewis et al. (1956); dt is the temporal step of simulation. The instantaneous [MeHg] in urine as function of time ([MeHgU(t)] ) is always equal to zero due to the parameter setting of our model. Therefore, the instantaneous [THg] in urine ([THgU]) is always equal to that of its inorganic form.

The instantaneous [MeHg] in hair ([MeHgH]) is obtained as follows:

MeHgH(t)=Hot-Hot-1WH∙dt   (S15)

where Hot and Hot-1 are the MeHg burdens in hair at time t and  t-1, respectively; WH is the daily weight growth of hair, which is fixed at 0.75 g∙d-1 in agreement with the experimental data collected in previous works (Slezak 2017; Wright 2018). The instantaneous [IHg] in hair ([IHgH]) is calculated similarly to methylated form. The [THg] in hair ([THgH]) is obtained as the sum of concentrations of the two Hg forms.

The instantaneous [MeHg] in faeces ([MeHgF]) is calculated as follows:

MeHgF(t)=Fot-Fot-1VF∙dt   (S16)

where Fot and Fot-1 are the MeHg burdens in faeces at time t and  t-1, respectively; VF is the daily volume growth of faeces, which is set at 0.5 l∙d-1 in accordance with values reported in previous works (NRC 2000; Brochot 2015). The instantaneous [IHg] in faeces ([IHgF]) is obtained similarly to methylated form. The [THg] in faeces ([THgF]) is calculated by summing the concentrations of the two Hg forms.

References

Ausili A, Gabellini M, Cammarata G, Fattorini D, Benedetti M, Pisanelli B, Gorbi S, Regoli F. Ecotoxicological and human health risk in a petrochemical district of southern Italy. Mar. Environ. Res. 2008; 66:215–217.

Brochot C. The Human MERLIN-Expo model, Fun Project 4 - Seventh Framework Programme 2015.

Brunet C, Casotti R, Vantrepotte V, Corato F, Conversano F. Picophytoplankton diversity and photoacclimation in the Strait of Sicily (Mediterranean Sea) in summer. I. Mesoscale variations. Aquat. Microb. Ecol. 2006; 44:127–141.

Brunet C, Casotti R, Vantrepotte V, Conversano F. Vertical variability and diel dynamics of picophytoplankton in the Strait of Sicily, Mediterranean Sea, in summer. Mar. Ecol. Prog. Ser. 2007; 346:15–26.

Campfens J, Mackay D. Fugacity-Based Model of PCB Bioaccumulation in Complex Aquatic Food Webs. Environ. Sci. Technol. 1997; 31:577-583.

Catalano D, Ientile R, Marletta A, Sciandrello S. Monitoraggio Biologico del SIC/ZPS “ITA090014 - Saline di Augusta” e della Foce del Fiume Mulinello nell’Ambito dei Lavori di Ampliamento del Porto Commerciale di Augusta (Terza Fase) – Fase Ante Operam. Relazione Tecnica, 2014.

Denaro G, Salvagio Manta D, Borri A, Bonsignore M, Valenti D, Quinci E, Cucco A, Spagnolo B, Sprovieri M, De Gaetano A. HR3DHG version 1: modeling the spatiotemporal dynamics of mercury in the Augusta Bay (southern Italy). Geosci. Model Dev. 2020; 13: 2073–2093. DOI: https://doi.org/10.5194/gmd-13-2073-2020.

Hendriks AJ, Van der Linde A, Cornelissen G, Sijm DTHM.The power of size. 1. Rate constants and equilibrium ratios for accumulation of organic substances related to octanol-water partition ratio and species weight. Environ. Toxicol. Chem. 2001; 20(7):1399-1420.

Hendriks AJ, Heikens A. The power of size. 2. Rate constants and equilibrium ratios for accumulation of inorganic substances related to species weight.  Environ. Toxicol. Chem. 2001; 20(7):1421-1437.

Lewis PR, Lobban MC, Shaw TI. Patterns of urine flow in human subjects during a prolonged period of life on a 22-hour day. J Physiol. 1956; 133:659-669. DOI: https://doi.org/10.1113/jphysiol.1956.sp005616.

McKinney RA, Glatt SM, Williams SR. Allometric length-weight relationships for benthic prey of aquatic wildlife in coastal marine habitats. Wildlife Biology 2004; 10(1):241-249.

NRC (National Research Council). Toxicologic effects of methylmercury. National Academy Press, Washington, DC, 2000. DOI: https://doi.org/10.17226/9899.

Radomyski A, Ciffroy P. The Phytoplankton MERLIN-Expo model, Fun Project 4 - Seventh Framework Programme, 2015a.

Radomyski A, Ciffroy P. The Invertebrate MERLIN-Expo model, Fun Project 4 - Seventh Framework Programme, 2015b.

Slezak JK, Ström JO, Theodorsson E. Testosterone-like immunoreactivity in hair measured in minute sample amounts - a competitive radioimmunoassay with an adequate limit of detection. Sci. Rep. 2017; 7:17636. DOI: https://doi.org/10.1038/s41598-017-17930-w.

Strickland JDH. Measuring the Production of Marine Phytoplankton, Fisheries Research Board of Canada (Bulletin), 1960.

Wright LP, Zhang LM, Cheng I, Aherne J, Wentworth GR. Impacts and Effects Indicators of Atmospheric Deposition of Major Pollutants to Various Ecosystems – A Review. Aerosol and Air Quality Research 2018; 18:1953–1992. DOI: https://doi.org/10.4209/aaqr.2018.03.0107.

Yakovlev VA, Yakovleva AV. Polychaete Hypania invalida (Polychaeta: Ampharetidae) in the Kuybyshev Reservoir: Distributionand Size and Weight Parameters. Russ. J. Biol. Invasions 2010; 1(2):145-151.

Tables

| Symbol | Interpretation | Unit | Value | Reference |
| --- | --- | --- | --- | --- |
| Dx | Horizontal turbulent diffusivity along x-axis | m2 h-1 | 3600.0 | Massel (1999) |
| Dy | Horizontal turbulent diffusivity along y-axis | m2h-1 | 3600.0 | Massel (1999) |
| Dz | Vertical turbulent diffusivity along z-axis | m2 h-1 | 1.2 | Denman at al. (1983) |
| vz | Vertical component of velocity field | m h-1 | 0.0 | Experimental data |
| kD | Water-SPM partition coefficient for inorganic mercury | l Kg-1 | 104.7 | Experimental data |
| kPh-de | Rate constant for the photo-demethylation of methylmercury | h-1 | 0.00216 | Monperrus et al. (2007) |
| kdeme | Rate constant for the biotic demethylation of methylmercury | h-1 | 0.014940 | Lehnherr et al. (2011) |
| kme | Rate constant for the methylation of inorganic mercury | h-1 | 0.000272 | Lehnherr et al. (2011) |
| vsilt | Silt settling velocity | m h-1 | 0.208 | Rosati et al. (2018) |
| kDsiltII | Partition coefficient of HgII to silt | l Kg-1 | 3000 | Rosati et al. (2018) |
| kDsiltMM | Partition coefficient of MeHg to silt | l Kg-1 | 1000 | Rosati et al. (2018) |
| H | Henry's law constant | dimensionless | 0.479 | Experimental data |
| uwind | Annual average wind speed | m s-1 | 3.83 | NASA website |
| PMCO2 | Molar mass of carbon dioxide | g mol-1 | 44.01 | Periodic table of the elements |
| PMHg0 | Molar mass of elemental mercury | g mol-1 | 200.59 | Periodic table of the elements |
| PMH2O | Molar mass of water | g mol-1 | 18.02 | Periodic table of the elements |
| λ | Mercury recycling coefficient for picoeukaryotes population | dimensionless | 0.52 | Valenti et al. (2017) |
| Dw-in | Molecular diffusion coefficient for inorganic mercury | m2 h-1 | 2.534·10-6 | Schulz and Zabel (2006) |
| Dw-or | Molecular diffusion coefficient for methylmercury | m2 h-1 | 2.534·10-6 | Schulz and Zabel (2006) |
| δsedII | Boundary layer thickness above sediment for inorganic mercury | m | 0.00009 | Calibration procedure |
| δsedMM | Boundary layer thickness above sediment for methylmercury | m | 0.00030 | Calibration procedure |
| δW | Boundary layer thickness below sediment | m | 0.0001 | Sørensen et al. (2001) |
| Kdemeth | Rate constant for the demethylation of methylmercury in the pore water of the sediment | h-1 | 0.007177 | Hines et al. (2012) |
| Kmeth | Rate constant for the methylation of inorganic mercury in the pore water of the sediment | h-1 | 0.000503 | Hines et al. (2012) |
| kMeHg | Average fraction of methylmercury in the sediment | dimensionless | 0.004 | Experimental data |
| KdII | Sediment - pore water distribution coefficient for inorganic mercury | l Kg-1 | 105.00 | Liu et al. (2012) |
| KdMM | Sediment - pore water distribution coefficient for methylmercury | l Kg-1 | 102.50 | Liu et al. (2012) |
| α | De-adsorption rate for the total mercury concentration in the sediment | h-1 | 1.0·10-7 | Calibration procedure |
| dep | SPM deposition rate at the seawater-sediment interface | mm y-1 | 11.7 | Sprovieri et al.(2015) |
| ρsed | Surface sediment density | g cm-3 | 2.6 | Ogrinc et al.(2007) |
| DepHgdryII | Atmospheric dry deposition of HgII | ng m-2 h-1 | 0.456 | Rajar et al.(2007) |
| HgDbou | Dissolved mercury concentration at the domain boundaries (Ionian sea) | ng l-1 | 0.26 | Horvat et al. (2003) |
| HgTbou | Total mercury concentration at the domain boundaries (Ionian sea) | ng l-1 | 0.31 | Horvat et al. (2003) |
| LLev | Levante inlet width | m | 400.0 | Salvagio Manta et al. (2016) |
| LSci | Scirocco inlet width | m | 300.0 | Salvagio Manta et al. (2016) |
| Qsource | Average flow rate of water for point source | m3 h-1 | 0.0 | Field observations |
| Hgsource0 | Elemental mercury concentration of point source | ng l-1 | 0.0 | Field observations |
| HgsourceII | Inorganic mercury concentration of point source | ng l-1 | 0.0 | Field observations |
| MeHgsource | Methylmercury concentration of point source | ng l-1 | 0.0 | Field observations |
| SLII | Direct loads of inorganic mercury | ng l-1 h-1 | 0.0 | Field observations |
| SLMM | Direct loads of methylmercury | ng l-1 h-1 | 0.0 | Field observations |

Tab. S1 – Environmental parameters used in the HR3DHG model. (Adapted from Denaro et al. [1]).

| Symbol | Interpretation | Unit | Range | Periodicity | Reference |
| --- | --- | --- | --- | --- | --- |
| vx | Horizontal velocity along x-axis | m h-1 | 0.0-0.5 | Three-hourly | Umgiesser (2009) |
| vy | Horizontal velocity along y-axis | m h-1 | 0.0-0.5 | Three-hourly | Umgiesser (2009) |
| forg | Organic fraction of suspended particulate matter in dissolved-phase | dimensionless | 0.004-0.010 | Constant | Experimental data |
| k1 | Rate constant for the photo-oxidation of elemental mercury | h-1 | 1.79-7.23 | Daily | Zhang et al. (2014) |
| k2 | Rate constant for the photo-reduction of inorganic mercury | h-1 | 0.43-1.75 | Daily | Zhang et al. (2014) |
| k3 | Rate constant for the biological oxidation of elemental mercury | h-1 | 0.00045-0.04830 | Three-hourly | Zhang et al. (2014) |
| k4 | Rate constant for the biological reduction of inorganic mercury | h-1 | 0.00028-0.02967 | Three-hourly | Zhang et al. (2014) |
| NPP | Net primary production | g C m-2 h-1 | 0.000386-0.007728 | Three-hourly | Baines et al.(1994) |
| Tatm | Surface atmospheric temperature | oC | 11.52-29.83 | Daily | NASA website |
| Hggas-atm | Gaseous mercury concentration in atmosphere | ng m-3 | 1.50-2.10 | Daily | Bagnato et al. (2013) |
| HgatmII | Inorganic mercury concentration in atmosphere | ng l-1 | 21.0-32.0 | Daily | Bagnato et al. (2013) |
| Pr | Daily amount of precipitation | mm | 0.41-3.33 | Monthly | NASA website |
| φsed | Porosity of the sediment | dimensionless | 0.18-0.56 | Constant | Experimental data |
| TOCsed | Total organic carbon at the sediment surface | percent dry weight | 0.210-4.780 | Constant | Experimental data |
| HgTsed | Initial total mercury concentration in sediment | mg Kg-1 | 0.005-300.000 | Constant | Sprovieri et al. (2011) |
| SPM | Suspended particulate matter conc. in seawater | mg l-1 | 18.4-31.0 | Constant | Experimental data |
| Rin | Nutrient concentration at the domain boundaries | mmol phos. m-3 | 0.010-0.100 | Constant | Experimental data |
| Iin | Incident light intensity at the water surface | µmol photons m-2 s-1 | 301.37-1524.30 | Daily | NASA website |
| chlacell | Chl-a cellular content of picoeukaryotes | fg chl-a cell-1 | 10.00-660.00 | Constant | Brunet et al. (2007) |
| zb | Depth of the water column | m | 0-30 | Constant | Sprovieri et al. (2015) |

Tab. S2 – Environmental variables used in the HR3DHG model. (Adapted from Denaro et al. [1]).

| Symbol | Interpretation | Unit | Value | Reference |
| --- | --- | --- | --- | --- |
| Vcell | Picoeukaryotes cell volume | m3 cell-1 | 14.00 | Experimental data |
| Wphy | Picoeukaryotes cell weight | g cell-1 | 0.000014 | Strickland (1960) |
| VFCII | Volume concentration factor for inorganic mercury | l Kg-1 | 27500 | Pickhardt and Fischer (2007) |
| VFCMM | Volume concentration factor for methylmercury | l Kg-1 | 800000 | Pickhardt and Fischer (2007) |
| agrowth | Intercept of phytoplankton growth rate | dimensionless | 0.22 | Ciffroy (2015) |
| bgrowth | Slope of phytoplankton growth rate | dimensionless | 0.15 | Ciffroy (2015) |
| k | Allometric rate exponent | dimensionless | 0.25 | Hendricks at al. (2007) |
| pcarbonphy | Organic carbon fraction of phytoplankton | dimensionless | 0.29 | Ciffroy (2015) |
| ρlipid | Lipid-layer permeation resistance | d | 32.0 | Hauck at al. (2011) |
| ρwater | Water-layer diffusion resistance | d | 0.0068 | Hauck at al. (2011) |
| blipid | Lipid permeation resistance exponent | dimensionless | 0.41 | Hendricks at al. (2001) |
| log10KIId | Water-Dissolved Organic Carbon partition coefficient for inorganic mercury | dimensionless | 5.4 | Allison and Allison (2005) |
| log10KMMd | Water-Dissolved Organic Carbon partition coefficient for methylmercury | dimensionless | 5.0 | Allison and Allison (2005) |
| abg | Background turbidity | m-1 | 0.060 | Valenti et al. (2017) |
| a1 | Average absorption coefficient of picoeukaryotes | m^2 mg chl-a -1 | 0.012 | Hickman et al. (2010) |
| a2 | Average absorption coefficient of phytoplankton >3 µm | m^2 mg chl-a -1 | 0.020 | Hickman et al. (2010) |
| w | Sinking velocity of picoeukaryotes | m h-1 | 0.000098 | Valenti et al. (2017) |
| r | Maximum specific growth rate of picoeukaryotes | h-1 | 0.096 | Raven et al. (2005) |
| mb | Specific loss rate of picoeukaryotes | h-1 | 0.010 | Veldhuis et al. (2005) |
| KI | Half-saturation constant of light-limited growth of picoeukaryotes | µmol photons m-2 s-1 | 67.50 | Valenti et al. (2017) |
| KR | Half-saturation constant of nutrient-limited growth of picoeukaryotes | mmol phosphorus m-3 | 0.200 | Valenti et al. (2017) |
| 1/Y | Nutrient content of picoeukaryotes | mmol phosphorus cell-1 | 2.00·10-12 | Timmermans et al. (2005) |
| ε | Nutrient recycling coefficient of picoeukaryotes | dimensionless | 0.52 | Valenti et al. (2017) |

Tab. S3 – Biological parameters used in the Phytoplankton model for mercury adsorption and the NP model. (Adapted from Denaro et al. [2]).

| Symbol | Interpretation | Unit | Value | Reference |
| --- | --- | --- | --- | --- |
| Wphy | Average phytoplankton cell weight | kg | 14*10-15 | Strickland (1960) |
| Wbiv | Average weight of bivalvia | kg | 4.3*10-3 | Radomyski and Ciffroy (2015) |
| Wamp | Average weight of amphipoda | kg | 38*10-6 | McKinney et al. (2004) |
| Wmys | Average weight of mysidacea | kg | 1.0*10-4 | Campfens and Mackay (1997) |
| Wdec | Average weight of decapoda | kg | 133*10-4 | McKinney et al. (2004) |
| Wpol | Average weight of polychaeta | kg | 162*10-6 | Yakovlev and Yakovleva (2010) |
| Wost | Average weight of osteichthyes | kg | 16*10-3 | Campfens and Mackay (1997) |
| κ | Rate exponent | dimensionless | 0.25 | Hendricks et al. (2001) |
| ρw | Water layer diffusion resistance | d kg-κ | 0.0028 | Hendricks et al. (2001) |
| ρw,ege | Water layer diffusion resistance for assimilation of chemicals from food | d kg-κ | 0.000011 | Hendricks et al. (2001) |
| ρlip,in | Lipid layer permeation influx resistance for respiratory uptake | d kg-κ | 0.21 | Hendricks and Heikens (2001) |
| ρlip,out | Lipid layer permeation efflux resistance | d kg-κ | 0.30 | Hendricks and Heikens (2001) |
| κpII | Lipid resistance exponents for inorganic mercury | dimensionless | 0.57 | Calibration procedure |
| κpMM | Lipid resistance exponents for methylmercury | dimensionless | 0.57 | Calibration procedure |
| γ0 | Water absorption-excretion coefficient | kgκ d-1 | 200 | Hendricks et al. (2001) |
| γ1 | Food ingestion coefficient | kgκ d-1 | 0.005 | Hendricks et al. (2001) |
| γ2 | Biomass reproduction coefficient | kgκ d-1 | 0.0006 | Hendricks et al. (2001) |
| KtwII | Dry tissue-water partition ratios for inorganic mercury | l kg-1 | 5500 | Calibration procedure |
| KtwMM | Dry tissue-water partition ratios for methylmercury | l kg-1 | 11000 | Calibration procedure |
| ps,phy | Dry fraction of phytoplankton | dimensionless | 0.0768 | Hendricks and Heikens (2001) |
| ps,biv | Dry fraction of bivalvia | dimensionless | 0.1698 | Hendricks and Heikens (2001) |
| ps,amp | Dry fraction of amphipoda | dimensionless | 0.1652 | Hendricks and Heikens (2001) |
| ps,mys | Dry fraction of mysidacea | dimensionless | 0.1517 | Hendricks and Heikens (2001) |
| ps,dec | Dry fraction of decapoda | dimensionless | 0.1714 | Hendricks and Heikens (2001) |
| ps,pol | Dry fraction of polycheata | dimensionless | 0.1715 | Hendricks and Heikens (2001) |
| ps,ost | Dry fraction of osteichthyes | dimensionless | 0.1788 | Hendricks and Heikens (2001) |
| qT | Temperature correction factor for cold-blooded animal | dimensionless | 1 | Hendricks et al. (2001) |
| pass_biv,HgII | Efficiency for dietary assimilation of inorganic mercury by bivalvia | dimensionless | 0.03 | Hendricks and Heikens (2001) |
| pass_amp,HgII | Efficiency for dietary assimilation of inorganic mercury by amphipoda | dimensionless | 0.16 | Hendricks and Heikens (2001) |
| pass_mys,HgII | Efficiency for dietary assimilation of inorganic mercury by mysidacea | dimensionless | 0.16 | Hendricks and Heikens (2001) |
| pass_dec,HgII | Efficiency for dietary assimilation of inorganic mercury by decapoda | dimensionless | 0.16 | Hendricks and Heikens (2001) |
| pass_pol,HgII | Efficiency for dietary assimilation of inorganic mercury by polychaeta | dimensionless | 0.01 | Calibration procedure |
| pass_ost,HgII | Efficiency for dietary assimilation of inorganic mercury by osteichthyes | dimensionless | 0.01 | Hendricks and Heikens (2001) |
| pass_fish,HgII | Efficiency for dietary assimilation of inorganic mercury by M. barbatus | dimensionless | 0.01 | Hendricks and Heikens (2001) |
| pass_biv,MeHg | Efficiency for dietary assimilation of methylmercury by bivalvia | dimensionless | 0.65 | Hendricks and Heikens (2001) |
| pass_amp,MeHg | Efficiency for dietary assimilation of methylmercury by amphipoda | dimensionless | 0.60 | Hendricks and Heikens (2001) |
| pass_mys,MeHg | Efficiency for dietary assimilation of methylmercury by mysidacea | dimensionless | 0.60 | Hendricks and Heikens (2001) |
| pass_dec,MeHg | Efficiency for dietary assimilation of methylmercury by decapoda | dimensionless | 0.60 | Hendricks and Heikens (2001) |
| pass_pol,MeHg | Efficiency for dietary assimilation of methylmercury by polychaeta | dimensionless | 0.99 | Calibration procedure |
| pass_ost,MeHg | Efficiency for dietary assimilation of methylmercury by osteichthyes | dimensionless | 0.88 | Hendricks and Heikens (2001) |
| pass_fish,MeHg | Efficiency for dietary assimilation of methylmercury by M. barbatus | dimensionless | 0.88 | Hendricks and Heikens (2001) |
| p'ass,biv | Fraction of ingested food assimilated by bivalvia | dimensionless | 0.4 | Hendricks et al. (2001) |
| p'ass,amp | Fraction of ingested food assimilated by amphipoda | dimensionless | 0.2 | Hendricks et al. (2001) |
| p'ass,mys | Fraction of ingested food assimilated by mysidacea | dimensionless | 0.4 | Hendricks et al. (2001) |
| p'ass,dec | Fraction of ingested food assimilated by decapoda | dimensionless | 0.8 | Hendricks et al. (2001) |
| p'ass,pol | Fraction of ingested food assimilated by polychaeta | dimensionless | 0.2 | Hendricks et al. (2001) |
| p'ass,ost | Fraction of ingested food assimilated by osteichthyes | dimensionless | 0.8 | Hendricks et al. (2001) |
| p'ass,fish | Fraction of ingested food assimilated by M. barbatus | dimensionless | 0.8 | Hendricks et al. (2001) |
| a | Initial growth coefficient | dimensionless | 0.009 | Calibration procedure |
| b | Allometric constant | dimensionless | 3.07 | Calibration procedure |
| L∞ | Asymptotic length of M. barbatus | mm | 235 | Tursi et al. (1996) |
| K | Growth coefficient of M. barbatus | y-1 | 0.275 | Tursi et al. (1996) |
| t'0 | Theoretical age of M. barbatus for length fixed to zero | y | -1.91 | Tursi et al. (1996) |

Tab. S4 - Biological parameters used in the INTFISH model. (Adapted from Denaro et al. [2]).

| Symbol | Interpretation | Unit | Value |
| --- | --- | --- | --- |
| fabs | Absorption fraction of MeHg ingested by humans | dimensionless | 0.97 |
| kabs | Oral dose absorption rate constant | d-1 | 5.544 |
| K | Proportionality constant between the whole human body organic Hg burden and the blood organic Hg burden | dimensionless | 12.987 |
| kBr | Partition coefficient between brain and blood organic Hg | dimensionless | 0.369 |
| kL | Partition coefficient between liver and blood organic Hg | dimensionless | 1.298 |
| kK | Partition coefficient between kidney and blood organic Hg | dimensionless | 0.115 |
| kR | Partition coefficient between rest of body and blood organic Hg | dimensionless | 7.000 |
| kQI | Metabolism rate constant of MeHg to IHg for men (women) | d-1 | 0.01347 (0.01700) |
| kQH | Organic Hg transfer coefficients from the whole body to hair | d-1 | 0.00244 |
| kQU | Organic Hg transfer coefficients from the whole body to urine | d-1 | 0.0 |
| kQF | Organic Hg transfer coefficients from the whole body to faeces | d-1 | 0.000145 |
| dBL | Blood to liver transfer coefficient combined with liver metabolism rate constant of MeHg for men (women) | d-1 | 0.175 (0.221) |
| dBBr | Blood to brain transfer coefficient combined with brain metabolism rate constant of MeHg | d-1 | 0.00055 |
| kLB | Transfer coefficient of IHg from liver to blood | d-1 | 2.57025 |
| kLF | Transfer coefficient of IHg from liver to faeces | d-1 | 0.46428 |
| kKB | Transfer coefficient of IHg from kidney to blood | d-1 | 0.0010 |
| kKU | Transfer coefficient of IHg from kidney to urine | d-1 | 0.006949 |
| kBK | Transfer coefficient of IHg from blood to kidney | d-1 | 2.140425 |
| kBBr | Transfer coefficient of IHg from blood to brain | d-1 | 0.0028 |
| kBH | Transfer coefficient of IHg from blood to hair | d-1 | 0.343 |
| kBU | Transfer coefficient of IHg from blood to urine | d-1 | 0.06994 |
| kBF | Transfer coefficient of IHg from blood to faeces | d-1 | 3.9917 |
| kBrB | Transfer coefficient of IHg from brain to blood | d-1 | 0.052 |

Tab. S5 – Parameters used in the BBD model.

| Age classes | EWIave (µg/KgBDW) | EWImax (µg/KgBDW) |
| --- | --- | --- |
| Toddlers (0-3 years) | 1.57 | 16.98 |
| Children (3-10 years) | 1.49 | 11.21 |
| Teenagers (10-18 years) | 1.09 | 7.99 |
| Adults (18-65 years) | 0.84 | 4.39 |
| Elderly (65-75 years) | 0.63 | 3.44 |
| Very Elderly (>75 years) | 0.35 | 3.44 |

Tab. S6 – Estimated weekly intake of MeHg for Italian population (EWIave) and local foodstuff consumers residing in the Augusta Bay (EWImax) calculated for the six age classes considering an exposure time of 365 days year-1.

| Period | Latitude | Longitude | Batch | Fish | % MeHg | % HgII |
| --- | --- | --- | --- | --- | --- | --- |
| May-12 | 37.1774 N | 15.2100 E | 1 | 1 | 88.7 | 11.3 |
| May-12 | 37.1774 N | 15.2100 E | 1 | 2 | 84.5 | 15.5 |
| May-12 | 37.1774 N | 15.2100 E | 1 | 3 | 88.7 | 11.3 |
| Oct-17 | 37.1857 N | 15.1998 E | 1 | 1 | 92.0 | 8.0 |
| Oct-17 | 37.1857 N | 15.1998 E | 1 | 2 | 92.1 | 7.9 |
| Oct-17 | 37.1857 N | 15.1998 E | 1 | 3 | 91.8 | 8.2 |
| Oct-17 | 37.1898 N | 15.1998 E | 2 | 1 | 91.0 | 9.0 |
| Oct-17 | 37.1898 N | 15.1998 E | 2 | 2 | 91.2 | 8.8 |
| Oct-17 | 37.1898 N | 15.1998 E | 2 | 3 | 91.3 | 8.7 |

Tab. S7 -Percentage of the MeHg and IHg concentration obtained by the integrated Fish model for each M. barbatus caught during the two sampling periods (May 2012 and October 2017). (Adapted from Denaro et al. [2])

Figures

Fig. S1 - Dynamics of the THg concentration in three M. barbatus caught in the Augusta Bay during the first sampling (May 2012). The blue lines show the THg concentration as a function of M. barbatus age obtained by the integrated Fish model. The red points are the THg concentrations measured in three caught M. barbatus. (Adapted from Denaro et al. [2]).

Fig. S2 – Dynamics of THg (black lines), MeHg (orange lines) and IHg (blue lines) concentration in blood (panel a), urine (panel b), hair (panel c) and faeces (panel d), during the life cycle of Italian male population. The numerical results obtained by the model are compared with the reference value (RV), the risk level I (HBMI or WHOI) and the risk level II (HBMII or WHOII) in blood, urine and hair fixed in previous studies. The values of the parameters are those reported in Table S5.

Fig. S3 – Dynamics of THg (black lines), MeHg (orange lines) and IHg (blue lines) concentration in blood (panel a), urine (panel b), hair (panel c) and faeces (panel d), during the life cycle of Italian female population. The numerical results obtained by the model are compared with the reference value (RV), the risk level I (HBMI or WHOI) and the risk level II (HBMII or WHOII) in blood, urine and hair fixed in previous studies. The values of the parameters are those reported in Table S5.

.

Fig. S4 – Dynamics of THg (black lines), MeHg (orange lines) and IHg (blue lines) concentration in kidney (panel a), brain (panel b), liver (panel c) and the whole body (panel d), during the life cycle of Italian male population. The model results are obtained by using the values of parameters reported in Table S5.

Fig. S5 – Dynamics of THg (black lines), MeHg (orange lines) and IHg (blue lines) concentration in kidney (panel a), brain (panel b), liver (panel c) and the whole body (panel d), during the life cycle of Italian female population. The model results are obtained by using the values of parameters reported in Table S5.