Abstract

| Title | Experimental investigation of transport properties of ion-exchange membranes equilibrated with highly concentrated multi-ionic solutions |  |
| --- | --- | --- |
| Authors | Giorgio Purpura1, Valeria Guenther2, Andrea Culcasi1, Alessandro Tamburini1, Andrea Cipollina1*, Christian J. Linnartz2, Matthias Wessling2, Giorgio Micale1 1 Dipartimento di Ingegneria, Università degli studi di Palermo (UNIPA) – Viale delle Scienze, Ed 6, 90128 Palermo, Italy; 2 Chemical Process Engineering AVT.CVT, RWTH Aachen University, Forckenbeckstraße 51, 52074 Aachen, Germany |  |
| Keyword(s) (max. 6) | Ion exchange membranes, Ionic diffusivities, Ionic conductivity, Multi-ionic solutions |  |
| Abstract | Ion-exchange membranes (IEMs) are a specific class of dense, polymeric membranes widely adopted in all electromembrane processes for the sustainable production of water, energy and chemicals and for the valorization of waste brines. In recent years, the pressing need for more efficient separation processes has pushed forward the development of highly selective membranes to recover specific trace elements from seawater or salt-lake water brines [1]. A deep and thorough knowledge of ionic transport properties of IEMs and how they are affected by IEM’s chemical structure and composition is required to produce tailor-made membranes. Recent works on the topic highlighted that at low external salt concentrations, species selectivity is mainly dominated by long-range electrostatic interactions between mobile ions and fixed charged groups in the polymeric matrix of the membranes. Therefore, by considering these interactions, thermodynamic and transport models have been proposed to predict partitioning coefficients and ionic diffusivities in the membrane phase [2]. However, most of these works focused on IEMs equilibrated with diluted and moderately concentrated single-salt solutions. Consequently, a fundamental understanding of how the transport properties of IEMs are affected by the system composition is still missing. This lack of fundamental knowledge is even more significant when dealing with highly concentrated brines. In such kind a system, long-range interactions among the charged species are screened greatly because of the high ionic strength, and the transport properties are likely to be mainly influenced by species-specific local interactions [3], [4]. Therefore, our research aims to investigate thoroughly the effects of concentration and composition on IEM transport properties. An extensive experimental campaign was carried out to determine partitioning coefficients, ionic diffusivities and membrane conductivity of a commercial homogeneous cation-exchange membrane equilibrated with multi-ionic solutions over a broad concentration range, including Na+, K+, Ca2+, Mg2+ and Cl-. These results provide valuable insights into the behaviour of different ions in IEMs, elucidate the effect of concentration and composition on IEMs transport properties and, through this understanding, pave the way for advancements in IEM design and modelling. |  |
| References (BibTex Format) | [1] | @title={ Environmental impact of direct lithium extraction from brines}, author={ M. L. Vera, W. R. Torres, C. I. Galli, A. Chagnes, and V. Flexer }, year={2023}, publisher={ Nat. Rev. Earth Environ.} } |
|  | [2] | @title={ Manning condensation in ion exchange membranes: A review on ion partitioning and diffusion models }, author={ D. Kitto and J. Kamcev }, year={2022}, publisher={ J. Polym. Sci } |
|  | [3] | @title={ Molecular and electrostatic origins of mixed salt partitioning phenomena in uncharged poly(ethylene oxide)-based membranes}, author={ E. S. Zofchak et al.}, year={2024}, publisher={ J. Memb. Sci.} |
|  | [4] | @title={ Modelling of selective ion partitioning between ion-exchange membranes and highly concentrated multi-ionic brines}, author={ G. Purpura et al.}, year={2024}, publisher={ J. Memb. Sci.} |

Acknowledgments. This work was supported by the EU within SEArcularMINE (Circular Processing of Seawater Brines from Saltworks for Recovery of Valuable Raw Materials) project – Horizon 2020 programme, Grant Agreement No. 869467. This output reflects only the author’s view. The European Health and Digital Executive Agency (HaDEA) and the European Commission cannot be held responsible for any use that may be made of the information contained therein.